# 3 Tasks

The following questions are to be answered within the scope of this project.

1. Determine the third-order differential equation that provides a mathematical relationship between the angular position φA of the motor and the motor voltage UR.

2. Determine a mathematical relationship between the position of the marker on the conveyor belt and the angular position φA.

3. Determine the transfer function in the Laplace domain that describes the displacement x of the conveyor belt marker as a function of the motor voltage UR. Verify the plausibility of the controlled system's behavior using a step response.

4. Sketch a single-loop control system with unit feedback, a PD controller, and the controlled system described above.

5. Design a PD controller using the root locus method (WOK) that meets the required settling time or overshoot. 6. Show the step response of the single-loop control system and the root locus of the open-loop control system G0 using a graph.

6. State the transfer function of the PD controller.

7. The lecture notes contain examples of implementing a controller based on an operational amplifier circuit. Determine realistic values ​​for capacitance (µF) and resistance (kΩ) according to the PD controller transfer function.

8. Due to its bandwidth, the operational amplifier has a finite transient response. This should be taken into account in the PD controller model for the single-loop control system. For this purpose, the transfer function of the PD controller is multiplied by a PT1 element of the form 1/(1+0.001s). Simulate the step response of the single-loop control system and change the time constant of the PT1 element until the desired control system behavior is achieved. What bandwidth does the operational amplifier require?

9. Design a digital PD controller for the above control system. Determine using the algorithms in the lecture notes a PD controller that exhibits the desired transient response. Vary the different performance criteria. Use a simulation time of 1 second. Graphically display the step response and the algorithm iteration error.

10. State the transfer function of the digital PD controller and determine according to the lecture notes a discrete-time controller function that can be used in a controller algorithm.

11. Provide the general program code for the determined PD controller (whether using the digital or analog PD controller).