# Setting Up QUARC Software

## Software Sources

QUARC Real-Time RapidControl: https://www.quanser.com/products/quarc-real-time-control-software/

Our software license covers up to version 4.2.65535.65535 (Quarc 2022 SP1).

NI Compact RIO (Maximum version 18.0 or 19.6): https://www.ni.com/de/support/downloads/drivers/download.ni-compactrio.html#

Visual Studio Community: https://visualstudio.microsoft.com/de/vs/community/

## Installation (in this exact order)

- Install Matlab with Simulink. Matlab must be version 2022b at most. For newer versions, a newer Quarc license is required. Simply install all components. License code is in the file FIK.txt and the network license file is network.lic.

2. Install Visual Studio Community for C++ Desktop (Version 17 was used).

3. Run the command 'mex –setup' in the command line for Matlab integration.

4. Install NI Compact RIO (Maximum version 18.0 or 19.6). Install completely.

5. Install QUARC Real-Time RapidControl Software (Quarc 2022 SP1). If the installation order was not followed, please perform the steps under 'Quarc_library not found' to register the library.

If the above order is followed, all dependencies should be fulfilled.

After successful installation, the included sample project in Simulink should look like this:

## Troubleshooting

### 'Quarc_library' not found

Solution according to: https://www.quanser.com/faq/im-getting-a-quarc_library-not-found-error-in-my-simulink-blocks/

The QUARC Real-Time Control software may not be installed or QUARC may have been installed before MATLAB. If QUARC was not installed, then close MATLAB and install the QUARC Real-Time Control software as instructed in the Quick Installation Guide. If QUARC was installed and MATLAB was installed after, then run the following commands:

cd(fullfile(getenv('QUARC_DIR'), 'quarc'));

quarc_setup;

This function adds the QUARC Targets library to the MATLAB path and updates the MATLAB documentation. It also performs other setup tasks to make QUARC fully functional in MATLAB and Simulink.

# Notes by Thomas Lønne Stiansen

## Toolchain not recognized / Installation Repair / License

Upon major updates of Windows, the Quanser programs may run into trouble. This makes it unable to recognize the toolchain selected in Simulink, which is solved by running the repair installation: 	C:\Global\Software\quarc2022sp1\quarc\install_quarc.exe

This repair has to be run as administrator, but may yet fail as the license path has to be updated. This is done by selecting the “change” option instead of “repair” after running the executable file (install_quarc.exe), then pasting the updated license path. As of writing this, the location is: C:\Global\Software\11920 QUARC Essentials Fh Vorarlberg.qlic

## Permission Denied

May occur if running the Simulink in a folder without sufficient permissions. Simply change current working directory to one you have permissions for :)